SB16_IDE.SYS   OS/2 driver version 1.02

=======================================

This document is viewed best with a nonproportional font (e.g. Courier)

1. Introduction

---------------

In 1995 Creative Technology Ltd. redesigned their sound cards in order

to replace all jumper settings by Intel/Microsoft's "plug and play"

software mechanism. If your computer's ROM contains a PnP BIOS there

should be no problem to use SoundBlaster PnP cards with OS/2. If your

computer has no PnP BIOS you need special software drivers to activate

your sound card after power on. While you can download official OS/2

drivers for the sound portion of SoundBlaster PnP cards, there exists

currently no support for the built-in IDE (CDRom) controller.

SB16_IDE.SYS is not an official OS/2 driver but can be considered

as a "temporary fix" to enable the IDE CDRom controller on Creative's

SoundBlaster PnP cards. It does not implement full PnP functions nor

does it check for hardware conflicts. It only verifies the presence of

the appropriate hardware and initializes the card's PnP registers to

predefined address settings. After loading this driver the SoundBlaster

card's IDE controller part can be recognized by IBM's standard

IBM1S506.ADD driver.

2. Installation

---------------

- Verify that the following I/O addresses and IRQs are free (RMVIEW.EXE):

I/O port 20Bh                       (for PnP configuration)

and I/O port 170h + port 376h + IRQ 15  (for IDE controller, option 2)

or          1E8h +      3EEh +     11  (       ...        ,  ...   3)

or          168h +      36Eh +     10  (       ...        ,  ...   4)

- Copy SB16_IDE.SYS to \OS2 or \OS2\BOOT of the OS/2 boot partition.

- Add one line to CONFIG.SYS:

BASEDEV=SB16_IDE.SYS 2          (exactly one space before number)

or BASEDEV=SB16_IDE.SYS 3

or BASEDEV=SB16_IDE.SYS 4

- Verify or update the following CONFIG.SYS entries:

BASEDEV=IBM1S506.ADD /V                      (option 2 only)

or BASEDEV=IBM1S506.ADD /V /A:1/PORT:1E8/IRQ:11 (option 3 only)

or BASEDEV=IBM1S506.ADD /V /A:1/PORT:168/IRQ:10 (option 4 only)

and BASEDEV=IBMIDECD.FLT

and DEVICE=<path>\OS2CDROM.DMD /Q

Exact syntax of options may be different depending on driver versions

and your hardware configuration.

- Verify that the CDRom drive's jumper is set to "Master" if it is the

only drive on the cable.

- Reboot

- If you want to install OS/2 from a CDRom connected to the SoundBlaster

card, you must include above CONFIG.SYS changes to the second boot

floppy disk. Include additionally the line "SET COPYFROMFLOPPY=1" to

CONFIG.SYS and copy SB16_IDE.SYS to the root directory of this floppy

disk (not yet tested).

3. Troubleshooting

------------------

- SB16_IDE.SYS has been tested with SoundBlaster16PnP and AWE32PnP,

but most likely it works with other SoundBlaster PnP family models.

It will definitely not work with compatible sound cards from other

manufacturers.

- SB16_IDE.SYS does not care about the type of CDRom drive connected

to the SoundBlaster card. You can only connect CDRom drives fully

compatible to the EIDE/ATAPI specification that are known to be

supported by IBM1S506.ADD.

- Do not load SB16_IDE.SYS if a PnP BIOS is present.

- Do not load SB16_IDE.SYS on future releases of OS/2 (>3.0)

if they contain generic PnP support.

- Check for I/O + IRQ conflicts, your computer may hang if there are

unresolved conflicts.

- During a ctrl-alt-del warm reboot the SoundBlaster PnP IDE port remains

active even without this driver.

- Without a true PnP BIOS its not possible to boot OS/2 from a secondary

hard disk connected to the SoundBlaster PnP card, SB16_IDE.SYS can't

help. It may be possible to access a hard disk connected to the PnP

card after OS/2 bas booted up.

- It is possible to change the predefined address of the PnP

configuration port from 20Bh by patching SB16_IDE.SYS at offset 21Ah.

(203h...3FFh step 4 allowed)

- For users of IBM's new hires timer driver TIMER0.SYS only: Because I

don't know if it reprograms the hardware timer chip already at load

time, I recommend to place BASEDEV=SB16_IDE.SYS in CONFIG.SYS before

the line BASEDEV=TIMER0.SYS.

4. Contact address and updates

------------------------------

- If you have problems or if you like to receive the source code,

send an e-mail to 100021.313@compuserve.com or RLalla@stepnet.de .

- You can check for an update (if any) called SB16_???.ZIP at

Internet:      ftp://hobbes.nmsu.edu/os2/drivers

Compuserve:    BLASTER forum section "3rd party software"

5. Copyrights

-------------

- SB16_IDE.SYS is FREEWARE. It has been written without any support or

permission from Creative Labs and/or IBM, use it at your own risk.

- "SoundBlaster","SoundBlaster16PnP", "PnP", "IBM", "OS/2" and

"WARP Connect" are trademarks or registered trademarks of their

respective owners.

6. Revision history

-------------------

- 1.0   Apr-30-1996 first release

- 1.01              private update only

- 1.02  Jul-02-1996 now works correctly after a warm reboot from DOS

Robert Lalla, Loerrach, Germany      Jul-04-1996