PPPOS2(1)                                               PPPOS2(1)

NAME

ppp - Point to Point Protocol for OS/2 TCP/IP 2.0

SYNOPSIS

ppp [ options ] [ com port ] [ speed ]

DESCRIPTION

The  Point-to-Point  Protocol  (PPP) provides a method for

transmitting datagrams over serial  point-to-point  links.

PPP is composed of three parts: a method for encapsulating

datagrams over serial links, an  extensible  Link  Control

Protocol  (LCP), and a family of Network Control Protocols

(NCP) for establishing and configuring different  network-

layer protocols.

The  encapsulation  scheme  is  provided by an OS/2 Ring 3

driver and an interface to the TCP/IP  kernel.   PPP  pro-

vides  the  basic  LCP, authentication support, and an NCP

for establishing and  configuring  the  Internet  Protocol

(IP) (called the IP Control Protocol, IPCP).

FREQUENTLY USED OPTIONS

<com port>

Communicate  over  the  named  com  device.   If no

device name is given, ppp will use com1.

<speed>

Set the async port line speed to <speed>.

asyncmap <map>

Set the async character map  to  <map>.   This  map

describes  which  control characters cannot be suc-

cessfully received over the serial line.  ppp  will

ask  the  peer to send these characters as a 2-byte

"escape" sequence.  The argument is a  32  bit  hex

number  with  each  bit representing a character to

escape.  Bit 0 (00000001) represents the  character

0x00;  bit  31  (80000000) represents the character

0x1f or ^_.  The default asyncmap is 0.  If  multi-

ple asyncmap options are given, the values are ORed

together.

auth   Require the  peer  to  authenticate  itself  before

allowing network packets to be sent or received.

connect <command>

Use  the  executable  or shell command specified by

<command> to set up the serial line.   This  script

would  typically  use  the  "slattach" program or a

REXX command file to dial the  modem  and  start  a

remote ppp session.

defaultroute

Add  a  default route to the system routing tables,

1

PPPOS2(1)                                               PPPOS2(1)

using the peer as the gateway, when  IPCP  negotia-

tion  is  successfully  completed.   This  entry is

removed when the PPP connection is broken.

file <filename>

Read options from file <filename>  (the  format  is

described below).

help (-?)

Display  ppp  usage information for the most common

options.

mru <n>

Set the MRU [Maximum Receive Unit] value to <n> for

negotiation.  ppp will ask the peer to send packets

of no more than <n> bytes.  The minimum  MRU  value

is 128.  The default MRU value is 1500.  A value of

296 is recommended for slow interactive  links  (40

bytes for TCP/IP header + 256 bytes of data).

netmask <n>

Set  the interface netmask to <n>, a 32 bit netmask

in "decimal dot" notation (e.g. 255.255.255.0).

passive

Enables the "passive" option in the LCP.  With this

option,  ppp will attempt to initiate a connection;

if no reply is received from  the  peer,  ppp  will

then  just  wait  passively  for a valid LCP packet

from the peer (instead of exiting, as it does with-

out this option).

restart

Restart  LCP  negotiation  after  link termination.

This overrides the default behavior of  terminating

the ppp driver after disconnect.

rtscts Use hardware flow control (i.e. RTS/CTS) to control

the flow of data on the serial port.

silent With this option, ppp will not transmit LCP packets

to  initiate  a connection until a valid LCP packet

is received from the peer.

OTHER OPTIONS

<local_IP_address>:<remote_IP_address>

Set the local and/or remote interface IP addresses.

Either one may be omitted.  The IP addresses can be

specified with a host  name  or  in  "decimal  dot"

notation   (e.g.  "150.203.23.247").   The  default

local address is the first IP address of  the  sys-

tem.   The remote address will be obtained from the

peer if not specified in any option.  Thus, in sim-

ple cases, this option is not required.

2

PPPOS2(1)                                               PPPOS2(1)

-all   Don't  request  or allow negotiation of any options

for LCP and IPCP, use default values.

-ac    Disable  Address/Control  compression   negotiation

(use default, i.e.  disabled).

-am    Disable  asyncmap  negotiation  (use  default, i.e.

0xffffffff).

-as <n>

Same as asyncmap <n>

-d     Increase debugging level.

-ip    Disable IP address negotiation.  With this  option,

the  remote  IP  address  must be specified with an

option on the command line or in an options file.

-mn    Disable  magic  number  negotiation.    With   this

option, ppp cannot detect a looped-back line.

-mru   Disable MRU [Maximum Receive Unit] negotiation (use

default, i.e. 1500).

-p     Same as the passive option.

-pc    Disable protocol field compression negotiation (use

default, i.e. disabled).

+ua <p>

Agree to authenticate using PAP [Password Authenti-

cation Protocol] if requested by the peer, and  use

the  data  in file <p> for the user and password to

send to the peer. The file contains the remote user

name, followed by a newline, followed by the remote

password, followed by a newline.

+pap   Require the peer to authenticate itself using  PAP.

-pap   Don't agree to authenticate using PAP.

+chap  Require  the peer to authenticate itself using CHAP

[Cryptographic Handshake  Authentication  Protocol]

authentication.

-chap  Don't agree to authenticate using CHAP.

-vj    Disable negotiation of Van Jacobson style IP header

compression (use default, i.e. no compression).

debug  Increase debugging level (same as -d ).

domain <d>

Append the domain name <d> to the local  host  name

3

PPPOS2(1)                                               PPPOS2(1)

for authentication purposes.  For example, if geth-

ostname() returns the name foobar,  but  the  fully

qualified  domain name is foobar.IBM.COM, you would

use the domain option to set  the  domain  name  to

IBM.COM.

exit   Causes  the  ppp  driver  to terminate on detecting

loss of the modem Data Carrier Detect (DCD) signal.

idle <n>

Set  the  amount of inactivity time (no IP traffic)

until the ppp driver hangs up the connection.

local  Don't use modem control lines.

name <n>

Set the name of the local system for authentication

purposes to <n>.

user <u>

Set  the  user  name to use for authenticating this

machine with the peer using PAP to <u>.

usehostname

Enforce the use of the hostname as the name of  the

local system for authentication purposes (overrides

the name option).

remotename <n>

Set the assumed  name  of  the  remote  system  for

authentication purposes to <n>.

proxyarp

Add  an entry to this system's ARP [Address Resolu-

tion Protocol] table with the  IP  address  of  the

peer  and  the  Token-Ring/Ethernet address of this

system.

login  Use the system password database for authenticating

the peer using PAP.

secret <secret>

Use  the secret <secret> for PAP and CHAP authenti-

cation with the peer.

lcp-restart <n>

Set the LCP restart interval (retransmission  time-

out) to <n> seconds (default 3).

lcp-max-terminate <n>

Set  the  maximum  number  of LCP terminate-request

transmissions to <n> (default 3).

4

PPPOS2(1)                                               PPPOS2(1)

lcp-max-configure <n>

Set the maximum  number  of  LCP  configure-request

transmissions to <n> (default 10).

lcp-max-failure <n>

Set   the  maximum  number  of  LCP  configure-NAKs

returned before starting to send  configure-Rejects

instead to <n> (default 10).

ipcp-restart <n>

Set the IPCP restart interval (retransmission time-

out) to <n> seconds (default 5 seconds).

ipcp-max-terminate <n>

Set the maximum number  of  IPCP  terminate-request

transmissions to <n> (default 3).

ipcp-max-configure <n>

Set  the  maximum  number of IPCP configure-request

transmissions to <n> (default 10).

ipcp-max-failure <n>

Set  the  maximum  number  of  IPCP  configure-NAKs

returned  before starting to send configure-Rejects

instead to <n> (default 10).

pap-restart <n>

Set the PAP restart interval (retransmission  time-

out) to <n> seconds (default 5 seconds).

pap-max-authreq <n>

Set  the maximum number of PAP authenticate-request

transmissions to <n> (default 10).

chap-restart <n>

Set the CHAP restart interval (retransmission time-

out  for challenges) to <n> seconds (default 5 sec-

onds).

chap-max-challenge <n>

Set the maximum number of CHAP challenge  transmis-

sions to <n> (default 10).

chap-interval <n>

If  this  option is given, ppp will rechallenge the

peer every <n> seconds.

OPTIONS FILES

Options can be taken from files as  well  as  the  command

line.   ppp  reads options from the files $ETC/ppp.cfg and

$HOME/ppprc  before  looking  at  the  command  line.   An

options  file  is parsed into a series of words, delimited

by whitespace.  Whitespace can be included in  a  word  by

enclosing  the word in quotes (").  A backslash (\) quotes

5

PPPOS2(1)                                               PPPOS2(1)

the following character.  A hash  (#)  starts  a  comment,

which continues until the end of the line.

AUTHENTICATION

The  PAP  and  CHAP secrets files can be setup to restrict

the set of IP addresses which individual users may use.

The default behaviour of ppp is to agree  to  authenticate

if  requested,  and to not require authentication from the

peer.  However, ppp will not agree to authenticate  itself

with  a  particular  protocol  if  it has no secrets which

could be used to do so.

Authentication is based on  secrets,  which  are  selected

from    secrets   files,   $ETC/pap.sct   for   PAP,   and

$ETC/chap.sct for CHAP.  Both secrets files have the  same

format,  and  both  can store secrets for several combina-

tions of server (authenticating  peer)  and  client  (peer

being  authenticated).  Note that ppp can be both a server

and client, and that different protocols can  be  used  in

the two directions if desired.

A secrets file is parsed into words similar to the options

file.  A secret is specified by a line containing at least

3 words, in the order client, server, secret.  Any follow-

ing words on the same line are  taken  to  be  a  list  of

acceptable  IP  addresses  for  that client.  If there are

only 3 words on the  line,  it  is  assumed  that  any  IP

address is OK.  To disallow all IP addresses, use "-".  If

the secret starts with an `@', what follows is assumed  to

be  the  name  of a file from which to read the secret.  A

"*" as the client or server name matches any  name  (wild-

card).  When selecting a secret, ppp takes the best match,

i.e. the match with the fewest wildcards.

A secrets file contains both secrets for use in  authenti-

cating  other  hosts,  plus  secrets which can be used for

authenticating ourselves to others.  Which secret  to  use

is  chosen  based  on  the  names  of the host (the `local

name') and its peer (the `remote name').  The  local  name

is set as follows:

if the usehostname option is given,

then  the  local  name  is the hostname of this machine

(with the domain appended, if supplied)

else if the name option is given,

then use the argument of the first name option seen

else if the local IP address is specified with a hostname,

then use that name

else  use the hostname of this machine (with the domain

appended, if given)

6

PPPOS2(1)                                               PPPOS2(1)

When  authenticating  ourselves using PAP, there is also a

`username' which is the local name by default, but can  be

set with the user option or the +ua option.

The remote name is set as follows:

if the remotename option is given,

then  use  the  argument  of the last remotename option

seen

else if the remote IP address is specified with a host-

name,

then use that host name

else the remote name is the null string "".

Secrets are selected from the PAP secrets file as follows:

o For authenticating the peer, look for a secret with

client ==

username  specified in the PAP authenticate-request, and

server == local name.

o  For authenticating ourselves to the peer, look  for  a

secret with

client == our username, server == remote name.

When authenticating the peer with  PAP,  a  secret  of  ""

matches  any  password supplied by the peer.  If the pass-

word doesn't match the secret, the password  is  encrypted

using  crypt()  and checked against the secret again; thus

secrets for authenticating  the  peer  can  be  stored  in

encrypted  form.   If  the login option was specified, the

username and password are also checked against the  system

password database.  Thus, the system administrator can set

up the pap-secrets file to allow PPP access only  to  cer-

tain  users,  and to restrict the set of IP addresses that

each user can use.

Secrets are selected from the CHAP secrets  file  as  fol-

lows:

o  For  authenticating the peer, look for a secret with

client == name

specified  in  the  CHAP-Response message, and server ==

local name.

o  For authenticating ourselves to the peer, look  for  a

secret with

client == local name, and server ==  name  specified  in

the CHAP-Challenge message.

Authentication  must  be  satisfactorily  completed before

IPCP (or  any  other  Network  Control  Protocol)  can  be

7

PPPOS2(1)                                               PPPOS2(1)

started.  If authentication fails, ppp will terminated the

link by closing LCP.  If IPCP negotiates  an  unacceptable

IP  address  for the remote host, IPCP will be closed.  IP

packets can only be sent or received when IPCP is open.

ROUTING

When IPCP negotiation is completed successfully, ppp  will

inform  the  TCP/IP  kernel  of  the  local  and remote IP

addresses for the ppp interface.  This  is  sufficient  to

create  a  host route to the remote end of the link, which

will enable the peers to exchange IP packets.   Communica-

tion  with other machines generally requires further modi-

fication to routing tables and/or ARP (Address  Resolution

Protocol) tables.  In some cases this may be done automat-

ically through the actions of command scripts, but in most

cases some further intervention is required.

Sometimes  it  is desirable to add a default route through

the remote host, as in the case of a  machine  whose  only

connection  to  the Internet is through the ppp interface.

The defaultroute  option  causes  ppp  to  create  such  a

default  route  when IPCP comes up, and delete it when the

link is terminated.

In some cases it is desirable to use proxy ARP, for  exam-

ple  on  a  server machine connected to a LAN, in order to

allow other hosts to communicate  with  the  remote  host.

The  proxyarp  option  causes  ppp  to  look for a network

interface on the same  subnet  as  the  remote  host,  (an

interface  supporting  broadcast  and ARP, which is up and

not a point-to-point or loopback  interface).   If  found,

ppp  creates  a permanent, published ARP entry with the IP

address of the remote host and the hardware address of the

network interface found.

EXAMPLES

In  the simplest case, you can connect the serial ports of

two machines via a NULL modem cable and  issue  a  command

like

ppp com1 38400 passive

to each machine.

The  process  of logging in to a host machine and starting

ppp can be automated by using the connect  option  to  run

slattach or a REXX script, for example:

ppp com1 38400 connect "slattach AT OK ATDT999-9999

CONNECT login: foobar Password: password"

If your serial connection is any more complicated  than  a

piece  of  wire,  you may need to arrange for some control

characters to be escaped.   In  particular,  it  is  often

8

PPPOS2(1)                                               PPPOS2(1)

useful  to  escape  XON (^Q) and XOFF (^S), using asyncmap

a0000.  If the path includes a telnet, you probably should

escape  ^]  as  well  (asyncmap  200a0000).   Don't use an

rlogin in the path - many implementations are  not  trans-

parent;  they  will remove the sequence [0xff, 0xff, 0x73,

0x73, followed by any 8 bytes] from the stream.

DIAGNOSTICS

Messages  are  logged  to  both  the   console   and   the

$ETC/ppp<n>.log file.

FILES

$ETC/ppp<n>.pid

Process-ID for ppp process on ppp interface unit n.

$ETC/pap.sct

Usernames,  passwords  and  IP  addresses  for  PAP

authentication.

$ETC/chap.sct

Names,  secrets and IP addresses for CHAP authenti-

cation.

$ETC/ppp.cfg

System default options for ppp,  read  before  user

default options or command-line options.

$HOME/ppprc

User  default  options,  read  before  command-line

options.

SEE ALSO

RFC1144

Jacobson, V.  Compressing TCP/IP headers  for  low-

speed serial links.  1990 February.

RFC1321

Rivest, R.  The MD5 Message-Digest Algorithm.  1992

April.

RFC1331

Simpson, W.A.  Point-to-Point  Protocol  (PPP)  for

the  transmission  of multi-protocol datagrams over

point-to-point links.  1992 May.

RFC1332

McGregor, G.  PPP Internet Protocol Control  Proto-

col (IPCP).  1992 May.

RFC1334

Lloyd, B.; Simpson, W.A.  PPP authentication proto-

cols.  1992 October.

9

PPPOS2(1)                                               PPPOS2(1)

NOTES

The following signals have the specified effect when  sent

to the ppp process.

SIGINT, SIGTERM

These  signals  cause ppp to terminate the link (by

closing LCP), restore the serial  device  settings,

and exit.

BUGS

Usability issues, several to be resolved prior to GA.

10